COMSATS University Islamabad, Lahore campus

COAL Mid Term Lab Exam

Submitted to: Sir Hamza Hamid

Submited by: Samiullah Ashfaq

SP24-BSE-107

Section A

# Question 1:

MOV AX, 0107H     ; Roll number value

MOV BX, 7010H     ; Reverse roll number value

MOV CX, 0C042H

ADD AX, BX

ADD BX, CX

SUB CX, AX

SUB BX, AX

After 1st add:

Step-by-Step Execution

1st ADD: ADD AX, BX

- AX = 0107H + 7010H = 7117H

- AX = 7117H

- BX = 7010H

Part (a) — After 1st  ADD instruction

AX = 7117H

| Flag | Value | Reason |
| --- | --- | --- |
| CF | 0 | 0107H + 7010H = 7117H which is ≤ FFFFh — result fits in 16 bits, no carry out of bit 15 → CF = 0 |
| ZF | 0 | Result 7117H ≠ 0000H, so the Zero Flag is NOT set → ZF = 0 |
| SF | 0 | MSB (bit 15) of 7117H = 0 (0111 0001 0001 0111), meaning the result is positive |

After 1st  sub:

Part (b) — After 1st  SUB instruction

1st SUB: SUB CX, AX

- CX = 0C042H − 7117H = 4F2BH

| Flag | Value | Reason |
| --- | --- | --- |
| OF | 1 | The result (4F2BH) has an MSB of 0, meaning it is positive. This "sign flip" indicates a signed overflow occurred. |
| AF | 1 | he sum of the lowest 4 bits is 1011. Since a borrow was required in the original subtraction, AF is set to 1. |
| PF | 1 | The result is 4F2BH. The low byte is 2BH. Binary of 2BH: 0010 1011. Count the number of 1s: There are four 1s. Since 4 is an even number, the Parity Flag is set to 1 (Even Parity). |

# Question 2:

# a. JE — Check if BL = 5

ORG 100h

MOV BL, 5           ; Input value

CMP BL, 5           ; Compare BL to 5

JE  is_equal        ; Jump if Zero Flag is set (BL - 5 = 0)

; If not equal

MOV AH, 2

MOV DL, 'N'         ; Display 'N' for Not Equal

INT 21h

JMP finish

is_equal:

MOV AH, 2

MOV DL, 'Y'         ; Display 'Y' for Equal

INT 21h

finish:

RET                 ; Return to OS

# b. Print Y if CL = 20, else N

ORG 100h

MOV CL, 20          ; Input value

CMP CL, 20          ; Compare CL to 20

JE  print_y         ; If CL == 20, jump to print_y

; Else: Print 'N'

MOV AH, 2

MOV DL, 'N'

INT 21h

JMP exit_prog       ; Jump over the 'Y' logic

print_y:

MOV AH, 2

MOV DL, 'Y'

INT 21h

exit_prog:

RET

# c. Display D to I using LOOP

ORG 100h

MOV CX, 6           ; Loop 6 times (D, E, F, G, H, I)

MOV DL, 'D'         ; Initial character 'D'

MOV AH, 2           ; DOS function to print character

char_loop:

INT 21h             ; Print the character currently in DL

INC DL              ; Move to next ASCII character (e.g., 'D' -> 'E')

LOOP char_loop      ; Decrement CX. If CX != 0, jump to char_loop

RET