Comsats University Islamabad, Lahore campus

# Assignment #3

COAL

# Submitted To:

Sir Hamza Hamid

# Submitted by:

Zain Yaqoob

Sp24-BSE-125

Section: A

Q1 
Part A.

PUSH STACK POINTER MOVEMENT TABLE

| Step | Instruction | SP Value | Explanation |
| --- | --- | --- | --- |
| 1 | Initial | FFFEH | Initial top of stack |
| 2 | PUSH AX | FFFCH | SP decreases by 2, stores 0125H |
| 3 | PUSH BX | FFFAH | SP decreases by 2, stores 5210H |

POP STACK POINTER MOVEMENT TABLE

| Step | Instruction | SP Value | Explanation |
| --- | --- | --- | --- |
| 1 | Before POP | FFFAH | Top contains 5210H |
| 2 | POP CX | FFFCH | SP increases by 2 |
| 3 | POP DX | FFFEH | SP returns to original value |

STACK MEMORY TABLE

| SS:FFFA | 5210H |
| --- | --- |
| SS:FFFC | 0125H |

Top of stack:

SP = FFFA

Part B : Ax And Bx

CX = AX AND BX   
CX = 0000

AX Or BX  
DX = AX OR BX

DX = 5335

AX XOR BX 
SI = AX XOR BX

SI = 5335

Part C : ROL DX, 3
DX = 0125H
After 1st ROL = 024AH

After 2nd ROL = 0494H

After 3rd ROL = 0928H
DX = 0928H

CF = 0

ROR DX, 2

DX = 0125H

After 1st ROR = 8092H

After 2nd ROR = 4049H
DX = 4049H

CF = 0

Part D :
5210H ÷ 1111H = 0004H

Remainder = 0DCCH

AX = Quotient

DX = Remainder

| Register | Value |
| --- | --- |
| AX | 0004H |
| DX | 0DCCH |

Part E :

STACK WORKING

Push phase

Stack contains: 5 -> 4  -> 3  -> 2 -> 1

Pop + Add

5 + 4 + 3 + 2 + 1 = 15

FINAL MEMORY RESULT

| Memory Address | Value |
| --- | --- |
| 0200H | 0005H |
| 0202H | 000FH |